# AW9-Pixel-Bridge — Physical-Device Field Test Report

Document purpose: this is a review packet for an independent reviewer. It reports what happened

when the shipped v1.2.2 release was installed and exercised on real Android hardware for the

first time, after five prior rounds in which every claim was based on JVM/Robolectric tests and CI.

Reviewer's job: check the diagnosis, challenge the severity ratings, and answer the explicit

questions in §9. Every claim in §5–§7 is backed by a command or log line that is reproduced so it can

be re-run.

Status of the codebase at the time of the field test: v1.2.2, tag v1.2.2, commit d9860c7,

115 unit tests passing, Android Lint 0 errors, both CI jobs green. None of that changed the

outcome below. The defects in §6 are fixed in `v1.2.3` (125 tests) and every fix was

re-verified on the same device — see §13.

• • •

# 1. Executive summary

The app **installs, launches, runs its foreground service, binds its HTTP server on a real LAN, and

authenticates correctly** on Android 15. Those parts are genuinely verified, not inferred.

Field testing also found four defects that 115 passing unit tests could not catch, one of them

serious:

**CRITICAL — a Health Connect write failure silently discards the entire telemetry update while the

webhook still answers {"status":"synced"}.** The dashboard then shows 0 BPM, 0 steps and no

battery even though the phone received and parsed valid data. A client (an iOS Shortcut, or the

watch) is told everything succeeded.

HIGH — malformed JSON is accepted as success (HTTP 200 {"status":"synced"}).

MEDIUM — any unrecognised path or method is treated as a telemetry POST, so GET / and

GET /unknown both return a "synced" telemetry response instead of 404/405.

LOW — `/control` reports `"status":"executed"` for commands it explicitly ignored.

Separately, the test device cannot grant Health Connect permissions at all (see §7). That is an

environment limitation, not an app defect, but it is what exposed defect 1.

The headline lesson: the write path's failure handling — not its happy path — is what the unit

tests never modelled. In JVM tests HealthConnectClient is null, so every write short-circuits and

the code after the write is never reached with a throwing client.

• • •

# 2. System under test

| Item | Value |
| --- | --- |
| Application | com.bridge.applewatch ("AW9 Pixel Bridge") |
| Version | versionName=1.2.2, versionCode=7 (verified on device via dumpsys package) |
| Artifact | release app-release.apk, 2,518,256 bytes, SHA-256 636859cd0694f238edb19d793ed355faefb37f94113b02653967b67939a7b2c3 |
| Install method | adb install -r (sideloaded; the release is debug-signed by an earlier documented decision) |
| minSdk / targetSdk | 34 / 35 |
| Source commit | d9860c7 (tag v1.2.2) |

• • •

# 3. Device and environment (all read from the device)

| Property | Value |
| --- | --- |
| Model | OnePlus 10T, CPH2419, device OP5552L1, manufacturer OnePlus |
| Android | 15 (ro.build.version.release=15), API 35 (ro.build.version.sdk=35) |
| Build | S.2479e09-16953-1bdd09, hardware qcom |
| Display | 1080 × 2412, density 480; the app renders in landscape on this device |
| Phone IP | 192.168.0.172/24 on wlan0 (Wi-Fi "TP-Link_40A8"); also a VPN on tun0 (100.124.30.77) |
| Host IP | 192.168.0.191 (Windows 11, Wi-Fi) — same /24, so traffic is genuinely over the LAN |
| Health Connect | controller from APEX: /apex/com.android.healthfitness/priv-app/HealthConnectControllerGoogle@372042540, versionCode=37, versionName=17; provider binder present as healthconnect: [android.health.connect.aidl.IHealthConnectService] |

Note on the device substitution: the project is written for a Pixel 9. This test ran on a OnePlus

10T because that was the attached device. Android-version-wise it is a *superset* of the target

(API 35 ≥ 34), so installability and platform APIs are representative. OEM-specific behaviour

(aggressive background management, permission UX) is not representative of a Pixel.

• • •

# 4. What changed on the device before testing

Permissions were granted partly by hand and partly through the real dialog, because the goal was to

reach the app's behaviour rather than to re-test Android's permission UI:

adb install -r app-release.apk                    # Success
adb shell am start -n com.bridge.applewatch/.MainActivity
adb shell pm grant com.bridge.applewatch android.permission.BLUETOOTH_SCAN
adb shell pm grant com.bridge.applewatch android.permission.BLUETOOTH_CONNECT
adb shell pm grant com.bridge.applewatch android.permission.ACCESS_FINE_LOCATION
adb shell pm grant com.bridge.applewatch android.permission.ACCESS_COARSE_LOCATION
adb shell pm grant com.bridge.applewatch android.permission.POST_NOTIFICATIONS

The app then started its foreground service (isForeground=true, foregroundId=1001, notification

channel aw9_pixel_bridge_channel) and began listening:

tcp6   0   0  [::]:8484   [::]:*   LISTEN

Health Connect permissions were not grantable — see §7.

• • •

# 5. Verified working (with evidence)

Each row was executed during this session against the device.

| # | Feature | Evidence |
| --- | --- | --- |
| W1 | Installs and launches on Android 15 without crashing | versionCode=7; process alive; empty logcat -b crash |
| W2 | Foreground service + persistent notification | dumpsys activity services → isForeground=true, channel aw9_pixel_bridge_channel |
| W3 | HTTP server binds and is reachable over the real LAN | from host: GET http://192.168.0.172:8484/status → HTTP 200 {"status":"online","bleConnected":false,"bpm":0,"hcSynced":0} |
| W4 | Endpoint discovery shows the correct live IP | UI: Wi-Fi (wlan0): http://192.168.0.172:8484/telemetry |
| W5 | Token auth: missing token rejected | curl -X POST .../telemetry → HTTP 401 |
| W6 | Token auth: correct token accepted | -H "X-Bridge-Token: fieldtest123" → HTTP 200 |
| W7 | Token auth: wrong token rejected | -H "X-Bridge-Token: wrong" → HTTP 401 {"status":"unauthorized","code":401} |
| W8 | Token auth: /status is also protected | with token set, GET /status without token → HTTP 401 |
| W9 | Token-in-body fallback works | {"authToken":"fieldtest123","steps":3000} → HTTP 200 |
| W10 | Auth badge reflects storage immediately | UI flipped to ● Token Auth Active (X-Bridge-Token required) on Save, and Cleared X-Bridge-Token (LAN-only mode) on Clear — this validates the v1.2.1 auth-badge fix on real hardware |
| W11 | 413 on oversized body | 300 KB body → HTTP 413 Payload Too Large; 250 KB (under the 256 KiB cap) → HTTP 200 |
| W12 | 431 on oversized header | 20,000-char header → HTTP 431 Request Header Fields Too Large |
| W13 | CORS preflight | OPTIONS with Origin/Access-Control-Request-Method → HTTP 204, Access-Control-Allow-Origin: * |
| W14 | Control: media play/pause | {"command":"MEDIA_PLAY_PAUSE"} → {"status":"executed","action":"Toggled Pixel 9 Media Play/Pause"} |
| W15 | Control: media next | MEDIA_NEXT → "Skipped to Next Track" |
| W16 | Control: camera shutter | CAMERA_SHUTTER → "Camera shutter broadcast attempted (no confirmation available)" |
| W17 | Control: find-phone alarm | FIND_MY_PIXEL → "Triggered Find My Pixel 9 Alarm (5s)"; UI Last Watch Control Action updated |
| W18 | BLE scanning starts | UI: Scanning BLE 0x004C Continuity & 0x180D GATT..., Continuity State: Passive Scan Active |
| W19 | Dashboard renders all metric cards | LIVE HEART RATE, HRV (RMSSD), STEPS TODAY, ACTIVE ENERGY, BLOOD OXYGEN (SpO2), DISTANCE & FLOORS, SPEED & CADENCE, RESTING HR & SLEEP all present with units |
| W20 | XML import entry point opens the system file picker | tapping Import export.xml → com.google.android.documentsui/.../PickActivity |

Not verified: whether the media keys actually changed playback (no music was playing, so there was

no observable media session), and whether the camera broadcast reached a camera app (the app itself is

honest that it cannot confirm this).

• • •

# 6. Defects found

## D1 — CRITICAL: a Health Connect write failure discards the whole telemetry update, and the webhook still reports success

Reproduction (this exact sequence was run):

# valid JSON, all metrics in range
curl -X POST http://192.168.0.172:8484/telemetry \
     -H "Content-Type: application/json" \
     --data-binary @payload.json
# -> {"status":"synced","hcTotal":0}

curl http://192.168.0.172:8484/status
# -> {"status":"online","bleConnected":false,"bpm":0,"hcSynced":0}

payload.json was:

{"heartRate":72,"steps":5200,"spo2":98.0,"activeCalories":310.5,"distanceMeters":2400.0,
 "floorsClimbed":7,"restingHeartRate":52,"respiratoryRate":14.5,"sleepMinutes":420,"battery":62}

The app's own in-app log (read from the UI) showed, at the same moments:

[23:46:49] Webhook JSON Error: android.health.connect.HealthConnectException:
           java.lang.SecurityException: Caller requires
           android.permission.health.WRITE_STEPS to write record type
           class android.health.connect.datatypes.StepsRecord

Both symptoms, one cause. In WatchBridgeService.kt:

L1086 — val writtenCount = healthManager.writeTelemetryPayload(payload, advance.delta)

L1108 — _telemetry.update { cur -> cur.copy(...) }  ← the UI state, after the write

L1125–1127 — catch (e: Exception) { logEvent("Webhook JSON Error: ${e.message}") }

L1006 — """{"status":"synced","hcTotal":${...}}"""  ← hardcoded, evaluated regardless of outcome

So when insertRecords throws (permission denied, provider unavailable, any HealthConnectException),

control jumps to the catch before _telemetry.update runs. Consequences:

The dashboard and /status keep their previous values — for a fresh install, permanently zero —

even though valid telemetry arrived and was parsed correctly.

The HTTP response is still 200 {"status":"synced"}, so the sender cannot detect the failure.

The log line says "Webhook JSON Error", which misattributes a *write* failure to the *JSON*, so

an operator reading the log is pointed at the wrong subsystem.

I confirmed the parser is not at fault by running the identical payload through

WebhookPayloadParser.parse in a JVM test: it correctly produced `heartRate=72, steps=5200, spo2=98.0,

activeCalories=310.5, distanceMeters=2400.0, floorsClimbed=7.0, restingHeartRate=52,

respiratoryRate=14.5, sleepMinutes=420, battery=62`.

Note on interaction with the previous round. v1.2.2 moved the *step baseline* commit to after the

write precisely so a failed write would not consume the delta (that fix works — the log shows

Step baseline established at 1234 only when the write path returned, and the delta was retained when

the write threw). The UI state update was not moved with it, so the same class of bug remained one

layer up.

Impact. The watch or a Shortcut receives a success acknowledgement for data that was never stored,

while the phone's dashboard shows nothing. This is silent health-data loss with a false positive

confirmation — the exact failure mode the project's own audits rated highest.

## D2 — HIGH: malformed JSON is accepted as success

POST /telemetry   body: {not json
-> HTTP 200  {"status":"synced","hcTotal":0}

JSONObject(rawJson) fails inside runCatching, the parser returns a default WebhookTelemetryPayload(),

and the handler returns "synced". A client sends a broken automation and is told it worked. Expected: 400.

## D3 — MEDIUM: unrecognised paths and methods are treated as telemetry POSTs

GET /          -> HTTP 200 {"status":"synced","hcTotal":0}
GET /unknown   -> HTTP 200 {"status":"synced","hcTotal":0}
POST /telemetry (empty body) -> HTTP 200 {"status":"synced","hcTotal":0}

The dispatch is when { GET && /status -> ...; POST && /control -> ...; else -> telemetry }, so the

else is a catch-all. Any path or method is answered as a successful telemetry ingest.

This was verified, not just read from the source. A GET to / carrying {"steps":9999} was

answered HTTP 200 {"status":"synced","hcTotal":0} and the app's log recorded a real ingest attempt

against Health Connect:

[23:49:21] Webhook JSON Error: ... SecurityException: Caller requires
           android.permission.health.WRITE_STEPS to write record type
           class android.health.connect.datatypes.StepsRecord

So an idempotent-looking GET is treated as a state-changing write. Expected: 404 for unknown

paths, 405 for wrong methods.

## D4 — LOW: `/control` reports `"executed"` for commands it ignored

{"command":"nonsense"} -> {"status":"executed","action":"Ignored Unknown Command: nonsense"}

The status field contradicts the action field. A client that checks only status concludes the

command ran. (The four commands the watch actually sends — MEDIA_PLAY_PAUSE, MEDIA_NEXT,

CAMERA_SHUTTER, FIND_MY_PIXEL — are all handled correctly, so this is a reporting defect only.)

• • •

## D5 — MEDIUM: the dashboard displayed readings the write layer rejected

Found during the re-test, not in the original field session.

POST /telemetry  {"heartRate":290,"spo2":100.1}   -> 200 (nothing written: out of range)
GET  /status                                      -> {"bpm":290}
Dashboard                                         -> "290 BPM"

buildTelemetryRecords correctly refuses a 290 bpm record — Health Connect never sees it —

but the display update only checked if (payload.heartRate > 0), so an impossible vital was

rendered as live data. The same applied to a 100.1 % SpO₂.

Fixed in `v1.2.3` by moving the bounds into one shared HealthMetricRanges object used

by both the write layer and the display, so they cannot drift apart again. Re-verified on the

device: {"heartRate":290} leaves the displayed value unchanged at 72.

# 7. CORRECTED: the empty consent sheet is a Health Connect layout defect, not an app defect

This section replaces an earlier and incorrect conclusion. The first version of this

report attributed the empty Health Connect consent sheet to the absence of the

com.google.android.apps.healthdata package and called it a device limitation. An

independent reviewer then proposed a different cause — a missing

android:permission="android.permission.START_VIEW_PERMISSION_USAGE" on the app's

VIEW_PERMISSION_USAGE activity, which would have made it an app-side manifest defect.

Both explanations are wrong, and this section records the controlled experiment that

settled it.

## What was observed originally

Requesting Health Connect permission rendered a consent sheet with **no data types listed

and a disabled "Allow" button**, so permission could not be granted by tapping:

'Allow AW9 Pixel Bridge to access Health Connect?'
'Choose data you want this app to read or write to Health Connect'
[ secondary_button "Don't allow" bounds=[72,927][392,1032] ]
[ primary_button_outline "Allow" bounds=[2003,927][2209,1032] ]   <-- disabled

Input was reaching the window (InputDispatcher logged the taps against

PermissionsActivity), so it was not a missed tap. The app declares all 18 health

permissions in its manifest and the packaged APK carries them (aapt2 dump permissions),

and it requests 18 — so it was not a missing-declaration problem either.

## The controlled experiment

The app was uninstalled (which resets Health Connect grants) and the **identical, unmodified

v1.2.2 APK** was reinstalled. No code was changed at any point in this comparison — only

the device orientation was varied.

Portrait (rotation 0) — the sheet is fully populated:

'Allow all'                                          enabled=true
'Allow "AW9 Pixel Bridge" to read'                   enabled=true
'Activity. 0 of 4 selected. Read Access'
'Active calories burned. Read Access. Off'
'Don't allow'                                        enabled=true
'Allow'                                              enabled=false (nothing selected yet)

Landscape (rotation 1) — on the very same dialog, the list disappears:

'Allow AW9 Pixel Bridge to access Health Connect?'
'Choose data you want this app to read or write to Health Connect'
'If you give read access, the app can read new data and data from the past 30 days'
'You can learn how AW9 Pixel Bridge handles your data in their privacy policy'
'Don't allow'  enabled=true
'Allow'        enabled=false

No permission rows, no "Allow all". Rotating back to portrait restores the list, so the

effect is deterministic and reversible.

## Confirmation that the dialog is fully functional

In portrait, tapping "Allow all" then "Allow" granted all 18 health permissions

(dumpsys package → 18 × granted=true), and the app immediately wrote to Health Connect

again:

POST /telemetry -> {"status":"synced","hcTotal":8}
GET  /status    -> {"status":"online","bpm":72,"hcSynced":8}

Therefore:

It is not a missing com.google.android.apps.healthdata package. On Android 14+ Health

Connect is the /apex/com.android.healthfitness mainline module, and

com.google.android.healthconnect.controller is the correct package; the Play Store

apps.healthdata app is the Android 13 backport and is *expected* to be absent on

Android 15. The provider binder (healthconnect: [IHealthConnectService]) and the

controller are both present and working.

It is not a missing START_VIEW_PERMISSION_USAGE manifest attribute. The unmodified

APK populates the full permission list and can be granted every permission.

It is an orientation-dependent rendering defect in Health Connect's own

PermissionsActivity on this device/OEM build. Nothing in this app's code can fix it.

Residual risk worth knowing: a user who rotates their phone to landscape while the

consent sheet is open cannot grant permission and gets no explanation. The app cannot detect

or work around this. It is worth reporting upstream, and it is a good argument for the app

surfacing "Health Connect permission not granted" prominently rather than relying on the

system dialog alone — which is exactly what v1.2.3 now does (a 403 plus a dashboard that

still shows the received data).

## Why this matters for the earlier defect

The ungranted-permission state — whether caused by this layout bug or simply by a user not

having authorised Health Connect yet, which is the normal first-run state — is precisely the

condition that exposed D1. So even though the layout defect is not the app's, the

handling defect it uncovered is.

# 8. What could not be tested, and why

Being explicit about the boundary of this session:

| Not tested | Reason |
| --- | --- |
| Apple Watch → iPhone → phone end-to-end | No Apple Watch or iPhone available. BLE scanning was active but nothing of type 0x004C/0x180D was in range, which is expected. |
| Any successful Health Connect write | Device cannot grant HC permission (§7). |
| Health Connect read-back / aggregate correctness | Same. |
| XML import completing | The system file picker opened, but synthetic input did not select a file reliably, so the import path was not exercised end-to-end. A test export.xml was pushed to /sdcard/Download/ ready for a manual run. |
| Actual effect of media-key commands | No media playing on the device, so no session state changed. |
| Battery drain, background survival over hours, OEM process killing | Out of scope for a single session; the OnePlus battery manager actively interfered (it raised a "Charging limit" modal that stole focus). |
| Rendered UI visual correctness | Verified structurally via uiautomator dumps and screenshots, not against a design. |

• • •

# 9. Questions for the reviewer

D1 root cause. Do you agree the correct fix is to commit the telemetry state update

independently of the Health Connect write, and to make the HTTP response reflect what actually

happened? Is there a reason to keep the update after the write?

D1 status code. On a failed Health Connect write, what should /telemetry return —

200 with an explicit {"status":"partial","hcWritten":0}, or a 5xx? The caller is often an iOS

Shortcut, which treats any 2xx as success and may not inspect the body.

D1 error classification. Should a SecurityException (permission not granted) be distinguished

from a transient provider failure, so the client knows whether to prompt the user or retry?

D2. Is 400 Bad Request correct for unparseable JSON, given that some legacy clients may post

text/plain? Should the content type be enforced?

D3. Should unknown paths return 404, and is there a reason the catch-all was written this way?

Should GET with a body ever be ingested?

D4. Should unhandled commands return 400/422 rather than 200 {"status":"executed"}?

§7. Is the disabled-Allow Health Connect screen explainable by the missing

com.google.android.apps.healthdata package, or is there an app-side cause I should investigate?

Anything missed. Given the evidence above, is there a plausible defect in these paths I have not

looked for?

• • •

# 10. Why the unit tests did not catch this

This is the most useful lesson from the session, and it is worth reviewing as a process question.

The 115 JVM tests construct HealthConnectManager with a null HealthConnectClient, because a

real client cannot be constructed off-device. Every write method therefore returns early

(val hc = client ?: return 0), and **the code after the write is never executed with a throwing

client**. The tests can prove the *gates* are right (and after the last round they genuinely are),

but they cannot prove the *error handling* is right.

Robolectric does not model Health Connect's permission enforcement, so SecurityException never

occurs in tests.

CI builds and lints the APK but never installs or runs it.

A future test that would have caught D1 without hardware: inject a HealthConnectClient that throws

SecurityException on insertRecords, then assert that (a) the telemetry state still updates and

(b) the HTTP status is not a success. That requires HealthConnectManager to accept a client

abstraction rather than calling HealthConnectClient.getOrCreate internally.

• • •

# 11. Reproduction appendix

adb install -r app-release.apk
adb shell am start -n com.bridge.applewatch/.MainActivity
adb shell pm grant com.bridge.applewatch android.permission.BLUETOOTH_SCAN
adb shell pm grant com.bridge.applewatch android.permission.BLUETOOTH_CONNECT
adb shell pm grant com.bridge.applewatch android.permission.ACCESS_FINE_LOCATION
adb shell pm grant com.bridge.applewatch android.permission.POST_NOTIFICATIONS

adb shell netstat -tln | grep 8484            # expect: tcp6 [::]:8484 LISTEN

# W3 / D1
curl -s http://192.168.0.172:8484/status
curl -s -X POST http://192.168.0.172:8484/telemetry \
     -H 'Content-Type: application/json' --data-binary @payload.json
curl -s http://192.168.0.172:8484/status       # still bpm:0

# D2
printf '{not json' > bad.json
curl -s -X POST http://192.168.0.172:8484/telemetry \
     -H 'Content-Type: application/json' --data-binary @bad.json   # -> 200 synced

# D3
curl -s http://192.168.0.172:8484/
curl -s http://192.168.0.172:8484/unknown

# D4
curl -s -X POST http://192.168.0.172:8484/control \
     -H 'Content-Type: application/json' -d '{"command":"nonsense"}'

# W11 / W12
head -c 300000 /dev/zero | tr '\0' 'x' > big.txt ; curl -s -o /dev/null -w '%{http_code}\n' \
     -X POST http://192.168.0.172:8484/telemetry --data-binary @big.txt   # 413

# Read the app's own log (this is where the real error surfaces)
adb shell uiautomator dump /sdcard/ui.xml && adb pull /sdcard/ui.xml .
# ... or read it from the UI: scroll to "Google Health Connect & Remote Control Receipts"

• • •

*Prepared from a live session against com.bridge.applewatch v1.2.2 on OnePlus 10T (CPH2419,

Android 15 / API 35). No claim in §5–§7 is inferred: each was produced by a command whose output is

quoted. The APK has still never run on a Pixel 9 and has still never exchanged data with a real Apple

Watch.*

• • •

# 12. Adjudication of the external forensic review

An independent reviewer produced a forensic adjudication of the first version of this report.

It upheld D1–D4, upgraded D3 to HIGH, proposed one new defect (D0) that it rated CRITICAL,

and proposed two further findings (D5, D6). Each claim was checked against the code and,

where a device could settle it, against the device. Verdicts:

| Claim | Reviewer's position | Verdict | Evidence |
| --- | --- | --- | --- |
| D1 failed write drops state + false 200 | CRITICAL, upheld | CONFIRMED | Reproduced on hardware; fixed in v1.2.3; re-verified (§13) |
| D2 malformed JSON = 200 | HIGH, upheld | CONFIRMED | {not json returned 200 synced; now 400 |
| D3 catch-all route ingests GET / | Upgraded to HIGH | CONFIRMED, upgrade justified | A GET / carrying {"steps":9999} was verified to attempt a StepsRecord write |
| D4 /control says "executed" on unknown verb | LOW, upheld | CONFIRMED | Returned {"status":"executed","action":"Ignored Unknown Command: …"}; now 400 |
| D0 empty HC consent sheet is an app manifest defect (missing START_VIEW_PERMISSION_USAGE) | Raised to CRITICAL, overturning §7 | FALSIFIED | Controlled A/B on the unmodified APK: the sheet is fully populated and grants all 18 permissions without that attribute. The real cause is landscape rendering (§7) |
| D5 hrSampleBuffer.clear() runs before the flush | HIGH (new) | FALSIFIED — already fixed | WatchBridgeService.kt:1134 snapshots without clearing and retries; samples are removed only after a confirmed write |
| D6 100.64.0.0/10 should be treated as private | LOW (new) | NOT A DEFECT | Deliberate and unit-tested exclusion; the peer check is defence in depth, not authentication |
| Claimed: pm grant can grant health permissions | "Bonus verification" | FALSIFIED | pm grant com.bridge.applewatch android.permission.health.WRITE_HEART_RATE throws; health permissions are granted only through the Health Connect consent flow |
| Claimed: absent com.google.android.apps.healthdata is a ROM defect | part of D0 reasoning | FALSIFIED | On Android 14+ Health Connect is the com.android.healthfitness APEX module; apps.healthdata is the Android 13 backport and is expected to be absent |

## What the reviewer got right, and it mattered

D1–D4 and the D3 severity upgrade are all correct. The D3 upgrade especially: the

original report rated it MEDIUM, and demonstrating that a GET could drive a write makes

HIGH the better call.

The testability recommendation was the most valuable single output of the review. It

proposed extracting a HealthDataSink interface and installing a throwing fake so the

service's failure handling becomes reachable from JUnit. That is exactly what v1.2.3

does, and it is why D1 can never regress silently now: a test throws SecurityException

from the sink and asserts the 403, the dashboard update and the step-delta retention.

The recommendation to decouple the display state from the write was correct and is

implemented, with one deliberate refinement: the reviewer suggested publishing state before

the write and keeping the *baseline* after it. That is exactly the split adopted — display

first, write, then commit the baseline only on success.

## Where the reviewer reasoned from stale or assumed code

The D5 claim describes code that does not exist in v1.2.2:

"Notice that hrSampleBuffer.clear() happens before flushHeartRateSamples(copy) is

called!"

The shipped code, unchanged since v1.2.1, is:

// Take a snapshot but do NOT clear it yet. `flushHeartRateSamples`
// returns 0 without throwing when Health Connect is unavailable, so
// draining first silently discarded every sample of the interval, once
// per 15 s, forever. Samples are removed only as they are confirmed.
val snapshot = synchronized(hrSampleBuffer) { hrSampleBuffer.toList() }
...
val count = healthManager.flushHeartRateSamples(snapshot)
if (count > 0) {
    synchronized(hrSampleBuffer) { hrSampleBuffer.removeAll(snapshot) }
}

So the defect was real *once*, was found and fixed in the previous round, and the review

re-reported it against a version that no longer contained it. This is the failure mode to

watch for when a reviewer works from a summary rather than from the repository.

The D0 claim is the more instructive one, because the reviewer supplied a confident

framework-level causal chain ("the APEX controller refuses to trust the rationale activity,

filters out all 18 requested permissions, and renders the sheet with 0 permissions") and also

*rejected* an environment explanation. Both turned out to be wrong, and a single two-minute

experiment settled it: reinstall the unmodified APK, open the sheet in portrait, observe that

all permissions are listed, tap "Allow all", observe all 18 granted. A plausible mechanism is

not evidence.

## Process conclusion

Three independent sources were wrong about §7 in three different ways — this report (missing

device package), the reviewer (missing manifest attribute), and the reviewer's rejection of

the environment explanation (the actual cause). The device was right. The lesson is not

"trust the device over reviewers"; it is that any claim about runtime behaviour should be

cheap to test, and should be tested, before it is written into a fix directive.

• • •

# 13. `v1.2.3` verification on the same device

All fixes were installed and re-tested on the OnePlus 10T before release. Health Connect

permissions were reset by uninstalling, so both the failure and success paths could be

exercised.

| Check | Expected | Observed |
| --- | --- | --- |
| POST /telemetry, HC permission denied | 403, dashboard still updates | HTTP 403 {"status":"error","code":"HEALTH_PERMISSION_DENIED","hcTotal":0} and /status → bpm:72 |
| POST /telemetry, HC permission granted | 200, records written | HTTP 200 {"status":"synced","hcTotal":8,"hcWritten":8} |
| Malformed JSON | 400 | HTTP 400 {"status":"bad_request","code":400,…} |
| Unknown path | 404 | HTTP 404 {"status":"not_found","code":404,…} |
| GET /telemetry | 405 + Allow | HTTP 405 {"status":"method_not_allowed","code":405,"allow":"POST, OPTIONS"} |
| POST /status | 405 | HTTP 405 … "allow":"GET, OPTIONS" |
| Unknown control command | 400 rejected | HTTP 400 {"status":"rejected","code":400,"action":"Ignored Unknown Command: nonsense"} |
| Valid control command | 200 executed | HTTP 200 {"status":"executed","action":"Triggered Find My Pixel 9 Alarm (5s)"} |
| {"heartRate":290} | not displayed | displayed value stayed at 72 |
| In-app log wording | honest attribution | Health Connect permission denied — grant it on this device: …, Webhook rejected: body is not a JSON object (9 bytes) |

Test suite: 125 tests (was 115), Android Lint 0 errors, both CI jobs green, release

v1.2.3 published.